Glycolysis Logic Gate Educational Kit

Step 1 of 10  ·  Glucose → Glucose-6-Phosphate (G6P)

Enzyme: Hexokinase  ·  ATP consumed: 1  ·  Net reaction: Glucose + ATP → G6P + ADP

## 1.  Biochemical Context

In Step 1 of glycolysis, Hexokinase phosphorylates glucose using one ATP molecule. The phosphate group is transferred from ATP to the C6 position of glucose, producing Glucose-6-Phosphate (G6P) and ADP. G6P is charged and membrane-impermeable — the cell has effectively 'trapped' the glucose. This step is irreversible under physiological conditions.

Three conditions must all be true for the reaction to proceed:

• Glucose is present (substrate available)

• ATP is present (energy source available)

• Hexokinase enzyme is active (catalyst present)

## 2.  Logic Signal Mapping

| Signal | Represents | Logic 1 (HIGH) | Logic 0 (LOW) |
| --- | --- | --- | --- |
| Glucose | Substrate input | Glucose present | No glucose |
| ATP | Energy input | ATP available | ATP depleted |
| Hexokinase | Enzyme enable | Enzyme active | Enzyme absent/inhibited |
| REACTION | AND gate output | Reaction occurs | Reaction blocked |
| G6P | Primary product | G6P produced | No G6P |
| ADP | By-product | ADP released | No ADP |
| LED / Trapped | Visual indicator | LED ON — glucose trapped | LED OFF |

## 3.  Boolean Logic Expression

The circuit implements the following logic:

REACTION  =  Glucose  AND  ATP  AND  Hexokinase

G6P       =  REACTION   (buffered)

ADP       =  REACTION   (buffered)

LED       =  REACTION   (drives LED — glucose trapped indicator)

All three inputs must be HIGH simultaneously. Any missing input (no glucose, no ATP, inactive enzyme) prevents the reaction. The single AND gate output fans out to three downstream paths: G6P output, ADP output, and LED visual indicator.

## 4.  Text-Based Schematic

Signal flow from left (inputs) to right (outputs). Each column is a logic stage.

| INPUTS |  | STAGE 1 |  | STAGE 2 |  | OUTPUTS |
| --- | --- | --- | --- | --- | --- | --- |
| Glucose ───── | ─ | ┐ |  |  |  |  |
| ATP ────────── | ─ | ┤ AND1 ├── | ── | ┐ |  |  |
| Hexokinase ── | ─ | ┘ |  | ┤ AND2 ├── | ── | ─┬─► [BUF]──► G6P |
|  |  | (AND1 out) | ── | ┘ |  | ├─► [BUF]──► ADP |
|  |  |  |  |  |  | └─► [LED]──► Trapped |

AND1 = pre-reaction gate (Glucose AND ATP). AND2 = reaction enable gate (AND1 output AND Hexokinase). AND2 output = REACTION signal, which fans out to three buffered outputs.

## 5.  Gate-Level Description

| Ref | Type | Input A | Input B | Function |
| --- | --- | --- | --- | --- |
| AND1 | 2-input AND | Glucose | ATP | Detects substrate + energy co-presence |
| AND2 | 2-input AND | AND1 out | Hexokinase | Full reaction enable: all three conditions met |
| BUF1 | Buffer | AND2 out | — | Drives G6P output — isolates load |
| BUF2 | Buffer | AND2 out | — | Drives ADP output — isolates load |
| LED1 | LED Driver | AND2 out | — | Visual 'glucose trapped' indicator |

## 6.  Suggested IC Implementation (74HC Series)

| IC Part | Package Function | Gates Used | Pins Used (example) |
| --- | --- | --- | --- |
| SN74HC08N | Quad 2-input AND | AND1, AND2 | Pins 1,2,3 (AND1); Pins 4,5,6 (AND2) |
| 74HC244 | Octal buffer / line driver | BUF1, BUF2 | Pins 2→18, 4→16 (outputs) |
| PN2222 or NPN BJT | LED transistor switch | LED1 driver | Base from AND2 via 1kΩ; collector → LED → Vcc |

Alternatively, replace BUF1/BUF2 with two more unused gates from the 74HC08 wired as unity buffers (both inputs tied together), saving a chip. LED current limiting resistor: R = (Vcc − Vf) / If, e.g. (5V − 2V) / 10mA = 300Ω → use 330Ω.

## 7.  ATP Counter Integration

Step 1 consumes 1 ATP. The ATP counter tracks net ATP across all 10 steps. The recommended implementation uses a presettable binary down-counter.

Recommended IC: 74HC193 (Presettable 4-bit Up/Down Counter)

Wiring for ATP counting:

AND2 (REACTION out) ──► CD (Count Down) input of 74HC193

QA..QD              ──► 4-bit BCD to 7-segment decoder (e.g. 74HC4511)

74HC4511            ──► Common-cathode 7-segment display

PRESET pins         ──► Set initial ATP count (e.g. 0b0010 = 2 for demo mode)

On each rising edge of the REACTION signal, the counter decrements by 1. A CARRY OUT (borrow) signal can trigger an 'ATP depleted' warning LED wired back into the ATP input of AND1 to lock out further reactions — a physically accurate model of cellular ATP depletion.

Parried with 7 segment decoders like 74LS47 or 74HC4511.

| Step | Reaction | ATP Change | Net ATP | Counter Value |
| --- | --- | --- | --- | --- |
| 1 | Glucose → G6P | −1 | −1 | Decrement |
| 2 | G6P → F6P | 0 | −1 | Hold |
| 3 | F6P → F1,6BP | −1 | −2 | Decrement |
| 4–5 | Splitting + isomerisation | 0 | −2 | Hold |
| 6–9 | Payoff phase (×2 each) | +4 | +2 | Increment ×4 |
| 10 | PEP → Pyruvate | +2 | +2 | Increment ×2 |

## 8.  Full Truth Table — Step 1

| Glucose | ATP | Hexokinase | REACTION | G6P | ADP | LED (Trapped) |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 | OFF |
| 1 | 0 | 0 | 0 | 0 | 0 | OFF |
| 0 | 1 | 0 | 0 | 0 | 0 | OFF |
| 0 | 0 | 1 | 0 | 0 | 0 | OFF |
| 1 | 1 | 0 | 0 | 0 | 0 | OFF |
| 1 | 0 | 1 | 0 | 0 | 0 | OFF |
| 0 | 1 | 1 | 0 | 0 | 0 | OFF |
| 1 | 1 | 1 | 1 ✓ | 1 | 1 | ON ✓ |

Only the final row (all inputs HIGH) triggers the reaction. This directly models the biological requirement for simultaneous substrate, energy, and enzyme availability.

## 9.  Design Notes & Pedagogical Callouts

Fan-out: The AND2 output drives three loads (BUF1, BUF2, LED driver). 74HC gates can source/sink ~25mA and drive 10 standard loads, so direct drive of buffers is fine. Use a transistor or dedicated LED driver for the LED to avoid loading the logic output.

Enzyme inhibition modelling: The Hexokinase input can be wired to a physical DIP switch. Flipping it LOW models G6P product inhibition — when G6P accumulates, it inhibits Hexokinase. Students observe the reaction gate block even when Glucose and ATP are present.

Power supply: Use a regulated 5V supply for 74HC series. If using 3.3V (e.g. from an RP2040/RP2350 companion board), 74LVC08 is the appropriate AND gate equivalent.

Next step: Step 2 — Glucose-6-Phosphate Isomerase converts G6P → Fructose-6-Phosphate (F6P). The G6P output of this circuit becomes an input to the Step 2 circuit. ATP counter holds (no change in Step 2).

Glycolysis Logic Kit · Step 1/10 · For educational use