Glycolysis Logic Gate Educational Kit

Step 10 of 10  ·  Phosphoenolpyruvate → Pyruvate + ATP

Enzyme: Pyruvate Kinase (PK)  ·  ATP change: +2 (x2)  ·  Net: PEP + ADP → Pyruvate + ATP  —  GLYCOLYSIS COMPLETE

## 1.  Biochemical Context

Step 10 is the final reaction of glycolysis, catalysed by Pyruvate Kinase (PK). The high-energy phosphate group from PEP is transferred to ADP, producing Pyruvate and ATP. The enol form of pyruvate that is initially produced spontaneously tautomerises to the far more stable keto form, releasing a large amount of free energy — this thermodynamic collapse makes the reaction essentially irreversible and drives it powerfully forward.

This is the second and final substrate-level phosphorylation of glycolysis. Like Step 7, it produces 2 ATP due to the x2 multiplier from the Step 4 split. The net result of the entire pathway is +2 ATP per glucose molecule, along with 2 NADH and 2 Pyruvate.

Pyruvate is the gateway metabolite — depending on oxygen availability it proceeds to the TCA cycle and oxidative phosphorylation (aerobic), or is converted to lactate or ethanol (anaerobic fermentation). Glycolysis itself is complete.

Three conditions must be true:

• PEP is present (product of Step 9)

• ADP is available (phosphate acceptor)

• Pyruvate Kinase is active

## 2.  The ATP Trilogy — Steps 1, 7, and 10

Step 10 completes the three-step ATP story of glycolysis. The circuit topology for all three is identical — a two-AND cascade with ADP as the second input to AND1 — making the structural symmetry obvious on the board.

| Step | Enzyme | AND1 inputs | ATP direction | LED label |
| --- | --- | --- | --- | --- |
| 1 | Hexokinase | Glucose + ATP | -1 (invest) | Trapped |
| 3 | PFK-1 | F6P + ATP | -1 (invest) | Committed / No Return |
| 7 | PGK | 1,3-BPG + ADP | +2 (recover) | ATP Made |
| 10 ◄ | Pyruvate Kinase | PEP + ADP | +2 (profit) | Complete |

Steps 1 and 3 spend ATP — their AND1 gates take ATP as an input. Steps 7 and 10 earn ATP — their AND1 gates take ADP as an input. This input swap is the single most important structural difference between the investment and recovery steps, and it is immediately visible on the circuit board.

## 3.  Logic Signal Mapping

| Signal | Represents | Logic 1 (HIGH) | Logic 0 (LOW) |
| --- | --- | --- | --- |
| PEP | Substrate (from Step 9) | PEP present | Step 9 not complete |
| ADP | Phosphate acceptor | ADP available | ADP depleted |
| Kinase | Pyruvate Kinase enable | PK active | Enzyme inhibited |
| REACTION | AND2 output | Final phosphorylation occurs | Reaction blocked |
| Pyruvate | Final product (x2) | Pyruvate produced | No product |
| ATP | Net energy gain (x2) | ATP produced — counter +2 | No ATP |
| Complete LED | Pathway completion signal | ON — glycolysis complete | OFF |

## 4.  Boolean Logic Expression

AND1      =  PEP  AND  ADP

REACTION  =  AND1  AND  Kinase           (AND2)

Pyruvate      =  REACTION   (BUF1)   [x2 — gateway to TCA / fermentation]

ATP           =  REACTION   (BUF2)   [x2 — counter increments +2, final value: +2]

Complete_LED  =  REACTION   (Q1 LED driver)

The ATP output from BUF2 routes to the Count Up (CU) pin of the 74HC193, same as Step 7. After this increment the display reads +2 — the net yield of the entire pathway from one glucose molecule.

## 5.  Text-Based Schematic

Identical cascade topology to Steps 1 and 7. AND1 → AND2 → three-way fan-out.

| INPUTS |  | STAGE 1 |  | STAGE 2 |  | OUTPUTS |
| --- | --- | --- | --- | --- | --- | --- |
| PEP (← Step 9) ───── | ─ | ┐ |  |  |  |  |
| ADP ──────────────── | ─ | ┤AND1├── | ── | ┐ |  |  |
| Kinase ───────────── | ─ | ┘ |  | ┤AND2├── | ── | ─┬─► [BUF1] ──► Pyruvate (x2) |
|  |  | (AND1→AND2) | ── | ┘ |  | ├─► [BUF2] ──► ATP (x2 → counter CU → final: +2) |
|  |  |  |  |  |  | └─► [Q1] ──► Complete LED |

Structurally identical to Steps 1 and 7. The only differences are the input labels (PEP instead of substrate, ADP instead of ATP for investment steps) and the LED label. Students who built Step 1 will recognise this circuit immediately.

## 6.  Gate-Level Description

| Ref | Type | Input A | Input B | Function |
| --- | --- | --- | --- | --- |
| AND1 | 2-input AND | PEP | ADP | High-energy substrate + acceptor check |
| AND2 | 2-input AND | AND1 out | Kinase | Full reaction enable — all three conditions met |
| BUF1 | Buffer | AND2 out | — | Drives Pyruvate output — terminal product, gateway to next pathway |
| BUF2 | Buffer | AND2 out | — | Drives ATP output — routes to 74HC193 CU pin, final counter increment |
| Q1 | NPN BJT (PN2222) | AND2 via 1kΩ | — | Complete LED driver — glycolysis pathway finished |

## 7.  Suggested IC Implementation (74HC Series, 5V rail)

| IC / Component | Function | Used for | Key connections |
| --- | --- | --- | --- |
| 74HC08 | Quad 2-input AND | AND1, AND2 | AND1: pins 1,2→3; AND2: pins 4,5→6; 2 gates spare |
| 74HC244 | Octal buffer | BUF1 (Pyruvate), BUF2 (ATP) | BUF2 also routes to 74HC193 CU — same wiring as Step 7 |
| PN2222A | NPN BJT | Q1 Complete LED | Base: 1kΩ from AND2 out; Collector: LED → 330Ω → Vcc; Emitter: GND |
| LED rainbow/RGB | Complete indicator | Q1 load | RGB or multicolour — the finale deserves a special LED |

## 8.  Complete Pathway Summary — All 10 Steps

| Step | Enzyme | Reaction | ATP | Net ATP | Gate struct. | LED label |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Hexokinase | Glucose → G6P | -1 | -1 | AND+AND | Trapped |
| 2 | PGI | G6P → F6P | 0 | -1 | AND | Isomerisation |
| 3 | PFK-1 | F6P → F-1,6-BP | -1 | -2 | AND+AND | Committed / No Return |
| 4 | Aldolase | F-1,6-BP → DHAP+G3P | 0 | -2 | AND | Split |
| 5 | TPI | DHAP → G3P | 0 | -2 | AND+AND+AND | G3P_A / Both Ready |
| 6 | GAPDH | G3P → 1,3-BPG | 0* | -2 | AND+AND+AND | Energy Captured |
| 7 | PGK | 1,3-BPG → 3-PG+ATP | +2 | 0 | AND+AND | ATP Made |
| 8 | PGM | 3-PG → 2-PG | 0 | 0 | AND | Relocated |
| 9 | Enolase | 2-PG → PEP+H2O | 0 | 0 | AND | High-Energy Created |
| 10 ◄ | Pyruvate Kinase | PEP → Pyruvate+ATP | +2 | +2 | AND+AND | Complete |

Final tally: 2 ATP invested (Steps 1+3), 4 ATP produced (Steps 7+10 x2 each), net +2 ATP per glucose. Plus 2 NADH (Step 6 x2) for oxidative phosphorylation downstream. 1 glucose molecule in, 2 pyruvate molecules out.

## 9.  Complete Bill of Materials — Full Kit

| Component | Qty (min) | Used in steps | Notes |
| --- | --- | --- | --- |
| 74HC08 (quad AND) | 4–5 | All steps | Share spare gates across adjacent steps — 1 chip can serve 2–3 steps |
| 74HC244 (octal buffer) | 3–4 | All steps | 8 channels per IC — share across multiple steps |
| PN2222A NPN BJT | ~15 | All LED drivers | One per LED; bulk buy cheaply |
| 74HC193 (up/down counter) | 1 | ATP display | Shared across whole kit — single counter tracks all steps |
| 74HC4511 (BCD decoder) | 1 | ATP display | Drives 7-segment display from counter output |
| 7-segment display | 1 | ATP display | Common-cathode; shows net ATP from -2 to +2 |
| LEDs (assorted colours) | ~15 | All steps | One per step output; colour-code by type (see Section 10) |
| Resistors 10kΩ | ~25 | All switch inputs | Pull-down resistors for every DIP switch input |
| Resistors 1kΩ | ~15 | All BJT bases | Base current limiters for LED drivers |
| Resistors 330Ω | ~15 | All LEDs | LED current limiters; recalculate if Vf differs from 2V |
| DIP switches | ~25 | All enzyme/cofactor inputs | One per switchable input across all 10 steps |
| 4x AA battery holder | 1 | Power | 6V → 78L05 → 5V regulated supply |
| 78L05 regulator | 1 | Power | TO-92 package; add 100nF bypass caps on input and output |

## 10.  Suggested LED Colour Map — Full Kit

| Step | LED label | Colour | Reason |
| --- | --- | --- | --- |
| 1 | Trapped | Green | Glucose locked in — first commitment |
| 2 | Isomerisation | Yellow | Shape change, no energy event |
| 3 | Committed | Orange | Resources spent, decision made |
| 3 | No Return | Red | Irreversible — pathway locked in |
| 4 | Split | White | Cleavage event — neutral |
| 4 | DHAP LED | Cyan | Indirect product — needs Step 5 |
| 4 | G3P LED | Blue | Direct payoff-phase product |
| 5 | G3P_A LED | Blue | Matches Step 4 G3P colour — same molecule |
| 5 | Both Ready | Green | Go signal — payoff phase begins |
| 6 | Energy Captured | Gold/Amber | First energy harvest |
| 7 | ATP Made | Bright green | First ATP produced — positive payoff |
| 8 | Relocated | Purple | Subtle positional change |
| 9 | High-Energy Created | White/UV | Most energetic intermediate |
| 10 | Complete | RGB/Rainbow | Glycolysis complete — celebrate! |

## 11.  ATP Counter — Step 10 and Final State

| Step | Reaction | ATP Change | Net ATP | Counter Action |
| --- | --- | --- | --- | --- |
| 1 | Glucose → G6P | -1 | -1 | Decrement |
| 2 | G6P → F6P | 0 | -1 | Hold |
| 3 | F6P → F-1,6-BP | -1 | -2 | Decrement |
| 4 | F-1,6-BP → DHAP+G3P | 0 | -2 | Hold |
| 5 | DHAP → G3P | 0 | -2 | Hold |
| 6 | G3P → 1,3-BPG+NADH | 0* | -2 | Hold (*NADH) |
| 7 | 1,3-BPG → 3-PG+ATP | +2 | 0 | Increment x2 |
| 8 | 3-PG → 2-PG | 0 | 0 | Hold |
| 9 | 2-PG → PEP+H2O | 0 | 0 | Hold |
| 10 ◄ | PEP → Pyruvate+ATP | +2 | +2 | INCREMENT x2 — FINAL: +2 |

## 12.  Full Truth Table — Step 10

| PEP | ADP | Kinase | REACTION | Pyruvate | ATP | Complete LED |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 | OFF |
| 1 | 0 | 0 | 0 | 0 | 0 | OFF |
| 0 | 1 | 0 | 0 | 0 | 0 | OFF |
| 0 | 0 | 1 | 0 | 0 | 0 | OFF |
| 1 | 1 | 0 | 0 | 0 | 0 | OFF |
| 1 | 0 | 1 | 0 | 0 | 0 | OFF |
| 0 | 1 | 1 | 0 | 0 | 0 | OFF |
| 1 | 1 | 1 | 1 ✓ | 1 | 1 | ON ✓ COMPLETE |

The final active row of the final truth table of the kit. All inputs HIGH, all outputs HIGH, Complete LED on, ATP counter at +2. Glycolysis is done.

## 13.  Design Notes & Pedagogical Callouts

The Complete LED as the kit's centrepiece: The RGB or rainbow LED on Step 10 should be the most visually prominent indicator on the board. When all 10 steps are active simultaneously and the Complete LED fires, the entire board is lit — a full visual representation of one glucose molecule being fully processed. Consider a brief celebration: the ATP counter showing +2 and every LED glowing is the intended finale moment.

Pyruvate Kinase regulation — the second major control point: Like PFK-1 in Step 3, Pyruvate Kinase is allosterically regulated. It is inhibited by ATP and alanine (signals of energy abundance), and activated by F-1,6-BP (feedforward activation from Step 3). Flipping the Kinase switch LOW while everything else is HIGH models the cell braking glycolysis when it is already energy-rich — a satisfying regulatory loop to demonstrate after the pathway has run to completion once.

The ADP input revisited: Both Step 7 and Step 10 take ADP as an input to AND1. Depleting ADP (flipping that switch LOW) stalls both ATP-producing steps simultaneously — the counter freezes at 0 and neither Complete nor ATP Made LEDs fire. This models the real metabolic consequence of ADP depletion and reinforces why ADP availability is a key regulator of glycolytic flux.

Running the full pathway: With all 10 step boards connected and all switches set to HIGH, a single rising edge entering Step 1 should propagate through the entire chain, lighting LEDs sequentially and incrementing the ATP counter at Steps 7 and 10. The propagation delay through 74HC gates is nanoseconds, so in practice all LEDs light nearly simultaneously — but students can observe the pipeline by toggling individual enzyme switches to pause the pathway at any step.

What comes next — Pyruvate's fate: Pyruvate leaving Step 10 has three destinations depending on conditions: (1) aerobic — enters mitochondria as Acetyl-CoA for the TCA cycle; (2) anaerobic in muscle — reduced to lactate to regenerate NAD+; (3) anaerobic in yeast — decarboxylated to acetaldehyde then reduced to ethanol. Each of these could be represented as a switchable output on a bonus Step 11 board for advanced classes.

Glycolysis Logic Kit · Step 10/10 · COMPLETE  ·  Net yield: +2 ATP, +2 NADH, +2 Pyruvate per glucose  ·  For educational use