Glycolysis Logic Gate Educational Kit

Step 4 of 10  ·  Fructose-1,6-Bisphosphate → DHAP + G3P

Enzyme: Aldolase  ·  ATP change: 0  ·  Net reaction: F-1,6-BP → DHAP + G3P

## 1.  Biochemical Context

Step 4 is catalysed by Aldolase, which cleaves the 6-carbon Fructose-1,6-Bisphosphate into two 3-carbon molecules: Dihydroxyacetone Phosphate (DHAP) and Glyceraldehyde-3-Phosphate (G3P). This is the structural turning point of glycolysis — the pathway splits from one molecule into two parallel tracks.

No ATP is consumed or produced. The reaction is the direct payoff of the double phosphorylation performed in Steps 1 and 3 — the two phosphate groups now anchor each 3-carbon fragment, one at each end of the original 6-carbon chain.

DHAP is not directly usable in the payoff phase. Step 5 (Triose Phosphate Isomerase) will convert it into a second molecule of G3P, effectively doubling the throughput from Step 6 onward. This is why all payoff-phase products are counted in pairs (x2).

This step is the first in the kit where a single input produces two distinct outputs — a fundamentally different circuit topology from Steps 1–3.

## 2.  Topology Change — Fan-Out Rather Than Cascade

| Feature | Steps 1 & 3 (Investment) | Step 4 (Aldolase) |
| --- | --- | --- |
| Gate structure | AND cascade (AND1 → AND2) | Single AND, then split |
| Input count | 3 (substrate + ATP + enzyme) | 2 (substrate + enzyme) |
| Output count | 2 logic + 1 LED (Step 1) / 2 LED (Step 3) | 2 logic + 3 LEDs |
| ATP change | -1 | 0 |
| Signal flow | Deepening cascade | Lateral split / fan-out |
| Key LED | Trapped / Committed / No Return | Split — one molecule becomes two |

## 3.  Logic Signal Mapping

| Signal | Represents | Logic 1 (HIGH) | Logic 0 (LOW) |
| --- | --- | --- | --- |
| F-1,6-BP | Substrate (from Step 3) | F-1,6-BP present | No substrate |
| Aldolase Enable | Enzyme enable | Aldolase active | Enzyme inhibited |
| REACTION | AND gate output | Cleavage occurs | Reaction blocked |
| Split LED | Cleavage event indicator | ON — molecule splitting | OFF |
| DHAP | Product 1 (upper fragment) | DHAP produced | No DHAP |
| DHAP LED | Product 1 visual | ON — DHAP present | OFF |
| G3P | Product 2 (lower fragment) | G3P produced | No G3P |
| G3P LED | Product 2 visual | ON — G3P present | OFF |

## 4.  Boolean Logic Expression

The circuit implements the following logic:

REACTION   =  F-1,6-BP  AND  Aldolase_Enable

Split_LED  =  REACTION          (Q1 — fires at the split point)

DHAP       =  REACTION          (BUF1 — upper branch)

DHAP_LED   =  REACTION          (Q2 — DHAP present indicator)

G3P        =  REACTION          (BUF2 — lower branch)

G3P_LED    =  REACTION          (Q3 — G3P present indicator)

All five outputs are driven from the same AND gate output. The Split LED fires first in the signal path (between the AND gate and the fan-out node), giving the visual impression of the cleavage happening before the two products appear — matching your schematic design exactly.

## 5.  Text-Based Schematic

Single AND gate fans out through a Split LED, then branches into two parallel product paths each with their own LED.

| INPUTS |  | GATE + SPLIT |  | OUTPUTS |
| --- | --- | --- | --- | --- |
| F-1,6-BP (← Step 3) ── | ─ | ┐ |  |  |
| Aldolase Enable ────── | ─ | ┤ AND ├─► [Q1/Split LED]─ | ─ | ─┬─► [BUF1] ──► DHAP (→ Step 5) |
|  |  | ┘ |  | ├─► [Q2] ──► DHAP LED |
|  |  |  |  | ├─► [BUF2] ──► G3P (→ Step 6) |
|  |  |  |  | └─► [Q3] ──► G3P LED |

The Split LED (Q1) sits inline between the AND output and the fan-out node. In practice this means Q1's collector output feeds the fan-out rail, so the LED illuminates as current flows through to the downstream loads. Alternatively Q1 can be wired as a parallel branch from the AND output alongside the fan-out — the visual effect is the same and the parallel wiring is simpler. See Section 7 for the recommended wiring.

## 6.  Gate-Level Description

| Ref | Type | Input | Output | Function |
| --- | --- | --- | --- | --- |
| AND1 | 2-input AND | F-1,6-BP, Aldolase En. | REACTION | Cleavage enable — substrate and enzyme both present |
| Q1 | NPN BJT (PN2222) | AND1 out via 1kΩ | Split LED | Cleavage event indicator — fires at the split point |
| BUF1 | Buffer | AND1 out | DHAP | Drives DHAP output to Step 5 |
| Q2 | NPN BJT (PN2222) | AND1 out via 1kΩ | DHAP LED | DHAP product present indicator |
| BUF2 | Buffer | AND1 out | G3P | Drives G3P output to Step 6 |
| Q3 | NPN BJT (PN2222) | AND1 out via 1kΩ | G3P LED | G3P product present indicator |

## 7.  Suggested IC Implementation (74HC Series, 5V rail)

| IC / Component | Function | Used for | Key connections |
| --- | --- | --- | --- |
| 74HC08 (shared) | Quad 2-input AND | AND1 | Use 1 spare gate from earlier step's 74HC08 if available; pins e.g. 9,10→8 |
| 74HC244 (shared) | Octal buffer | BUF1 (DHAP), BUF2 (G3P) | Two channels; remaining channels spare |
| PN2222A (x3) | NPN BJT | Q1 Split, Q2 DHAP, Q3 G3P | Each: Base 1kΩ from AND1 out; Collector: LED → 330Ω → Vcc; Emitter: GND |
| LED white (x1) | Split indicator | Q1 load | White — neutral cleavage event, no product bias |
| LED cyan/teal (x1) | DHAP indicator | Q2 load | Cool colour — DHAP is the indirect product (needs Step 5) |
| LED blue (x1) | G3P indicator | Q3 load | Blue — G3P goes directly into the payoff phase |

Fan-out note: AND1 now drives five loads — BUF1, BUF2, Q1, Q2, Q3. Each BJT base draws ~2.5mA via 1kΩ (at 5V), so three BJTs draw ~7.5mA total, plus negligible buffer input current. This is still within the 74HC08 output current rating (~25mA). No buffer amplification needed on the AND output.

Split LED wiring clarification: Wire Q1 as a simple parallel branch directly from AND1 out — same node as BUF1, BUF2, Q2, Q3 base resistors. Do not place Q1 in series with the fan-out rail, as this would cause a voltage drop that affects downstream logic levels.

## 8.  LED Colour Rationale — Step 4

| LED | Suggested Colour | Pedagogical Reason |
| --- | --- | --- |
| Split | White | Neutral — the event itself, before either product is named |
| DHAP | Cyan / Teal | Cool tone — DHAP is a dead-end fragment that needs converting in Step 5 before it can continue |
| G3P | Blue | Distinct from DHAP — G3P proceeds directly into the payoff phase at Step 6 |

Having three LEDs illuminate simultaneously with different colours gives students an immediate visual: one molecule in, three lights out — the split is tangible. The white Split LED being physically positioned before the DHAP/G3P LEDs on the board reinforces the sequence of events.

## 9.  ATP Counter — Step 4

No ATP is consumed or produced. The counter holds at -2 (net), the lowest point of the pathway. No connection from Step 4 logic to the counter clock is required.

| Step | Reaction | ATP Change | Net ATP | Counter Action |
| --- | --- | --- | --- | --- |
| 1 | Glucose → G6P | -1 | -1 | Decrement |
| 2 | G6P → F6P | 0 | -1 | Hold |
| 3 | F6P → F-1,6-BP | -1 | -2 | Decrement |
| 4 ◄ | F-1,6-BP → DHAP + G3P | 0 | -2 | HOLD ◄ |
| 5 | DHAP → G3P | 0 | -2 | Hold |
| 6-9 | Payoff phase (x2 each) | +4 | +2 | Increment x4 |
| 10 | PEP → Pyruvate | +2 | +2 | Increment x2 |

## 10.  Full Truth Table — Step 4

| F-1,6-BP | Aldolase En. | REACTION | DHAP | G3P | Split LED | DHAP+G3P LEDs |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | OFF | OFF |
| 1 | 0 | 0 | 0 | 0 | OFF | OFF |
| 0 | 1 | 0 | 0 | 0 | OFF | OFF |
| 1 | 1 | 1 ✓ | 1 | 1 | ON ✓ | ON ✓ |

The simplest truth table in the kit so far — two inputs, one active row. Row 2 (substrate present, no enzyme) models aldolase inhibition. Row 3 (enzyme active, no substrate) models the scenario where Step 3 has not yet completed — Step 4 is waiting on the pipeline.

## 11.  Design Notes & Pedagogical Callouts

The split as a visual metaphor: The physical layout of this step on the board should reflect the biology — one wire coming in from Step 3, then visibly forking into two output connectors going to Steps 5 and 6. If the boards are arranged linearly, Step 4 is the junction where the single chain branches. Consider a Y-shaped PCB footprint or connector arrangement for this step.

DHAP vs G3P distinction: Both LEDs light simultaneously, but their destinations are different. DHAP goes to Step 5 (isomerisation to G3P), while G3P goes directly to Step 6. A label on the board indicating these different destinations reinforces that the two fragments are not equivalent in terms of pathway flow.

Why the Split LED matters: Without the Split LED, students see the AND gate output and two product LEDs — the cleavage event itself is invisible. The Split LED makes the act of breaking the bond explicit and observable, which is the core biochemical event of this step.

Doubling effect: This is a good moment to introduce the x2 multiplier for all payoff-phase outputs. From this step forward, every ATP produced is produced twice — once from each G3P molecule. The ATP counter should reflect this from Step 6 onward.

Next step: Step 5 — Triose Phosphate Isomerase converts DHAP into a second molecule of G3P. It is the simplest step in the kit: one input, one output, no ATP change, and it effectively doubles the G3P supply feeding into the payoff phase.

Glycolysis Logic Kit · Step 4/10 · For educational use